clc;

clear;

close all;

% ECA13 - EXPERIMENT 2

% MICROWAVE TWO-PORT NETWORK ANALYSIS

% Numerical Simulation and Performance Analysis of

% Microwave Transmission Lines and Two-Port Networks

% Using Scattering Parameters

% PARAMETERS

c = 3e8;

Z0 = 50;

L = 50e-3;

f0 = 10e9;

alpha = 0.6;

er_eff = 2.0;

% FREQUENCY SWEEP: 8 GHz TO 12 GHz

f = 8e9:10e6:12e9;

fg = f/1e9;

N = length(f);

% TRANSMISSION LINE PARAMETERS

beta = 2*pi*f/c*sqrt(er_eff);

x = (f-f0)/(2e9);

% ILLUSTRATIVE FREQUENCY-DEPENDENT MODEL

S11_mag = 10.^((-40 + 10*x.^2)/20);

S22_mag = S11_mag;

IL_dB = 0.3 + 0.2*x.^2;

S21_mag = 10.^(-IL_dB/20);

S12_mag = S21_mag;

% COMPLEX S-PARAMETERS

S11 = S11_mag;

S22 = S22_mag;

S21 = S21_mag .* exp(-1j*beta*L);

S12 = S12_mag .* exp(-1j*beta*L);

% S-PARAMETER MAGNITUDES IN dB

S11_dB = 20*log10(abs(S11));

S22_dB = 20*log10(abs(S22));

S21_dB = 20*log10(abs(S21));

S12_dB = 20*log10(abs(S12));

% RETURN LOSS AND INSERTION LOSS

RL_dB = -20*log10(abs(S11));

IL = -20*log10(abs(S21));

% TRANSMISSION EFFICIENCY

efficiency = abs(S21).^2*100;

% CENTER FREQUENCY INDEX

[~,idx] = min(abs(f-f0));

% CREATE FIGURE

figure('Name','Microwave Two-Port Network Analysis', ...

'Color','w','Position',[50 50 1400 750]);

sgtitle(['ECA13 - EXPERIMENT 2: ' ...

'MICROWAVE TWO-PORT NETWORK ANALYSIS'], ...

'FontWeight','bold','FontSize',13);

% 1. INPUT REFLECTION S11

subplot(3,3,1);

plot(fg,S11_dB,'r','LineWidth',2);

hold on;

plot(fg(idx),S11_dB(idx),'ko','MarkerFaceColor','r');

title('Input Reflection S11');

xlabel('Frequency (GHz)');

ylabel('S11 (dB)');

grid on;

xlim([8 12]);

ylim([-42 -28]);

% 2. FORWARD TRANSMISSION S21

subplot(3,3,2);

plot(fg,S21_dB,'r','LineWidth',2);

hold on;

plot(fg(idx),S21_dB(idx),'ko','MarkerFaceColor','k');

title('Forward Transmission S21');

xlabel('Frequency (GHz)');

ylabel('S21 (dB)');

grid on;

xlim([8 12]);

ylim([-0.55 -0.25]);

% 3. REVERSE TRANSMISSION S12

subplot(3,3,3);

plot(fg,S12_dB,'m','LineWidth',2);

title('Reverse Transmission S12');

xlabel('Frequency (GHz)');

ylabel('S12 (dB)');

grid on;

xlim([8 12]);

ylim([-0.55 -0.25]);

% 4. OUTPUT REFLECTION S22

subplot(3,3,4);

plot(fg,S22_dB,'k','LineWidth',2);

title('Output Reflection S22');

xlabel('Frequency (GHz)');

ylabel('S22 (dB)');

grid on;

xlim([8 12]);

ylim([-42 -28]);

% 5. RETURN LOSS

subplot(3,3,5);

plot(fg,RL_dB,'b','LineWidth',2);

title('Return Loss');

xlabel('Frequency (GHz)');

ylabel('RL (dB)');

grid on;

xlim([8 12]);

ylim([28 42]);

% 6. INSERTION LOSS

subplot(3,3,6);

plot(fg,IL,'r','LineWidth',2);

title('Insertion Loss');

xlabel('Frequency (GHz)');

ylabel('IL (dB)');

grid on;

xlim([8 12]);

ylim([0.25 0.55]);

% 7. TRANSMISSION EFFICIENCY

subplot(3,3,7);

plot(fg,efficiency,'Color',[0.2 0.75 0.4], ...

'LineWidth',2);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)');

grid on;

xlim([8 12]);

% 8. PHASE CONSTANT

subplot(3,3,8);

plot(fg,beta,'Color',[0.2 0.5 0.8], ...

'LineWidth',2);

title('Phase Constant');

xlabel('Frequency (GHz)');

ylabel('Beta (rad/m)');

grid on;

xlim([8 12]);

% 9. SIMULATION SUMMARY

subplot(3,3,9);

axis off;

summaryText = {

'SIMULATION SUMMARY'

'--------------------------'

sprintf('Z0 = %.0f Ohm',Z0)

sprintf('Length = %.0f mm',L*1000)

sprintf('Center Frequency = %.2f GHz',f(idx)/1e9)

sprintf('S11 = %.2f dB',S11_dB(idx))

sprintf('S21 = %.2f dB',S21_dB(idx))

sprintf('S12 = %.2f dB',S12_dB(idx))

sprintf('S22 = %.2f dB',S22_dB(idx))

sprintf('Return Loss = %.2f dB',RL_dB(idx))

sprintf('Insertion Loss = %.2f dB',IL(idx))

sprintf('Efficiency = %.2f %%',efficiency(idx))

sprintf('Beta = %.2f rad/m',beta(idx))

sprintf('Alpha = %.2f Np/m',alpha)

};

text(0.02,0.98,summaryText, ...

'Units','normalized', ...

'VerticalAlignment','top', ...

'FontName','FixedWidth', ...

'FontSize',9);

% COMMAND WINDOW OUTPUT

fprintf('\n========================================\n');

fprintf(' MICROWAVE TWO-PORT NETWORK ANALYSIS\n');

fprintf('========================================\n');

fprintf('Characteristic Impedance = %.2f Ohm\n',Z0);

fprintf('Transmission Line Length = %.2f mm\n',L*1000);

fprintf('Center Frequency = %.2f GHz\n',f(idx)/1e9);

fprintf('S11 = %.2f dB\n',S11_dB(idx));

fprintf('S21 = %.2f dB\n',S21_dB(idx));

fprintf('S12 = %.2f dB\n',S12_dB(idx));

fprintf('S22 = %.2f dB\n',S22_dB(idx));

fprintf('Return Loss = %.2f dB\n',RL_dB(idx));

fprintf('Insertion Loss = %.2f dB\n',IL(idx));

fprintf('Transmission Efficiency = %.2f %%\n',efficiency(idx));

fprintf('Beta = %.2f rad/m\n',beta(idx));

fprintf('Alpha = %.2f Np/m\n',alpha);

fprintf('========================================\n');

OP

Insertion Loss = 0.30 dB
Transmission Efficiency = 93.33 %
Beta = 296.19 rad/m
Alpha = 0.60 Np/m
========================================

>>